Hackathon Report

Smart Vehicle ECU Validation Framework

Group C14

| Sl.No. | Member Name |
| --- | --- |
| 1 | Gobinda Dey |
| 2 | Om Prasad |
| 3 | Suchandra Das |
| 4 | Pushkar Aditya |

Contents

| Sl.No. | Topic | Page No. |
| --- | --- | --- |
| 1 | Integration Test Report | 2 |
| 2 | Code Performance (Per Test Case) | 3–7 |
| 3 | Module-wise Aggregate Summary | 8 |
| 4 | Justification of Performance Results | 9–10 |

# 1.   Integration Test Report

| TC | Test Condition | Expected Outcome | Observed Outcome | Status |
| --- | --- | --- | --- | --- |
| TC1 | Normal Start – No faults | NORMAL status, no alerts | 0 alerts, SYSTEM STATUS: NORMAL | PASS |
| TC2 | Engine overheat (115 °C) | CRITICAL ENGINE OVERHEAT alert | 1 CRITICAL alert, STATUS: CRITICAL | PASS |
| TC3 | Low battery (10.0 V) | LOW BATTERY WARNING | 1 WARNING alert, STATUS: WARNING | PASS |
| TC4 | Overspeed (121 km/h) | OVERSPEED WARNING | 1 WARNING alert, STATUS: WARNING | PASS |
| TC5 | Door open while moving (11 km/h) | DOOR OPEN WARNING | 1 WARNING alert, STATUS: WARNING | PASS |
| TC6 | Seatbelt unlocked (1 km/h) | SEATBELT WARNING | 1 WARNING alert, STATUS: WARNING | PASS |
| TC7 | Low tire pressure (24.99 PSI) | LOW TIRE PRESSURE on FL | 1 WARNING alert, STATUS: WARNING | PASS |
| TC8 | All non-critical alerts combined | 4 WARNINGs, WARNING status | 4 WARNING alerts, STATUS: WARNING | PASS |
| TC9 | All alerts (engine + all sensors) | 6 alerts, CRITICAL status | 1 CRITICAL + 5 WARNINGs, STATUS: CRITICAL | PASS |
| TC10 | Boundary value verification | Door + seatbelt warnings only | 2 WARNINGs, STATUS: WARNING | PASS |

# 2.   Code Performance (Per Test Case)

## Tick 1: Normal operation

| Engine Temp | Battery | Speed | Tire Press. | Door | Seatbelt |
| --- | --- | --- | --- | --- | --- |
| 95.0 °C | 12.5 V | 50 km/h | 32 PSI | CLOSED | LOCKED |

Active Alerts: None  |  System Status: NORMAL  |  Execution Time: 0.027 ms  |  Validation: PASS

| Module | CPU Cycle | Code Size | Data Size |
| --- | --- | --- | --- |
| SensorManager | 624 | 4 KB | 128 B |
| AlertManager | 886 | 8 KB | 256 B |
| TOTAL | 1510 | 12 KB | 384 B |

## Tick 2: Engine overheat

| Engine Temp | Battery | Speed | Tire Press. | Door | Seatbelt |
| --- | --- | --- | --- | --- | --- |
| 115.0 °C | 12.5 V | 50 km/h | 32 PSI | CLOSED | LOCKED |

Active Alerts: 1  |  System Status: CRITICAL  |  Execution Time: 0.052 ms  |  Validation: PASS

[CRITICAL]  CRITICAL ENGINE OVERHEAT

| Module | CPU Cycle | Code Size | Data Size |
| --- | --- | --- | --- |
| SensorManager | 814 | 4 KB | 128 B |
| AlertManager | 1432 | 8 KB | 256 B |
| TOTAL | 2246 | 12 KB | 384 B |

## Tick 3: Low battery

| Engine Temp | Battery | Speed | Tire Press. | Door | Seatbelt |
| --- | --- | --- | --- | --- | --- |
| 90.0 °C | 10.0 V | 50 km/h | 32 PSI | CLOSED | LOCKED |

Active Alerts: 1  |  System Status: WARNING  |  Execution Time: 0.031 ms  |  Validation: PASS

[WARNING]  LOW BATTERY WARNING

| Module | CPU Cycle | Code Size | Data Size |
| --- | --- | --- | --- |
| SensorManager | 600 | 4 KB | 128 B |
| AlertManager | 1226 | 8 KB | 256 B |
| TOTAL | 1826 | 12 KB | 384 B |

## Tick 4: Overspeed

| Engine Temp | Battery | Speed | Tire Press. | Door | Seatbelt |
| --- | --- | --- | --- | --- | --- |
| 90.0 °C | 12.0 V | 121 km/h | 32 PSI | CLOSED | LOCKED |

Active Alerts: 1  |  System Status: WARNING  |  Execution Time: 0.03 ms  |  Validation: PASS

[WARNING]  OVERSPEED WARNING

| Module | CPU Cycle | Code Size | Data Size |
| --- | --- | --- | --- |
| SensorManager | 720 | 4 KB | 128 B |
| AlertManager | 854 | 8 KB | 256 B |
| TOTAL | 1574 | 12 KB | 384 B |

## Tick 5: Door open while moving

| Engine Temp | Battery | Speed | Tire Press. | Door | Seatbelt |
| --- | --- | --- | --- | --- | --- |
| 90.0 °C | 12.0 V | 11 km/h | 32 PSI | OPEN | LOCKED |

Active Alerts: 1  |  System Status: WARNING  |  Execution Time: 0.03 ms  |  Validation: PASS

[WARNING]  DOOR OPEN WARNING

| Module | CPU Cycle | Code Size | Data Size |
| --- | --- | --- | --- |
| SensorManager | 526 | 4 KB | 128 B |
| AlertManager | 908 | 8 KB | 256 B |
| TOTAL | 1434 | 12 KB | 384 B |

## Tick 6: Seatbelt unlocked while moving

| Engine Temp | Battery | Speed | Tire Press. | Door | Seatbelt |
| --- | --- | --- | --- | --- | --- |
| 90.0 °C | 12.0 V | 1 km/h | 32 PSI | CLOSED | UNLOCKED |

Active Alerts: 1  |  System Status: WARNING  |  Execution Time: 0.032 ms  |  Validation: PASS

[WARNING]  SEATBELT WARNING

| Module | CPU Cycle | Code Size | Data Size |
| --- | --- | --- | --- |
| SensorManager | 682 | 4 KB | 128 B |
| AlertManager | 840 | 8 KB | 256 B |
| TOTAL | 1522 | 12 KB | 384 B |

## Tick 7: Low tire pressure (BEST PERFORMANCE – fewest CPU cycles)

| Engine Temp | Battery | Speed | Tire Press. | Door | Seatbelt |
| --- | --- | --- | --- | --- | --- |
| 90.0 °C | 12.0 V | 40 km/h | 24.99 PSI | CLOSED | LOCKED |

Active Alerts: 1  |  System Status: WARNING  |  Execution Time: 0.029 ms  |  Validation: PASS

[WARNING]  LOW TIRE PRESSURE on FL

| Module | CPU Cycle | Code Size | Data Size |
| --- | --- | --- | --- |
| SensorManager | 550 | 4 KB | 128 B |
| AlertManager | 854 | 8 KB | 256 B |
| TOTAL | 1404 | 12 KB | 384 B |

## Tick 8: All non-critical alerts

| Engine Temp | Battery | Speed | Tire Press. | Door | Seatbelt |
| --- | --- | --- | --- | --- | --- |
| 95.0 °C | 9.0 V | 130 km/h | 20 PSI | CLOSED | UNLOCKED |

Active Alerts: 4  |  System Status: WARNING  |  Execution Time: 0.066 ms  |  Validation: PASS

[WARNING]  LOW BATTERY WARNING

[WARNING]  OVERSPEED WARNING

[WARNING]  LOW TIRE PRESSURE on FL

[WARNING]  SEATBELT WARNING

| Module | CPU Cycle | Code Size | Data Size |
| --- | --- | --- | --- |
| SensorManager | 852 | 4 KB | 128 B |
| AlertManager | 1534 | 8 KB | 256 B |
| TOTAL | 2386 | 12 KB | 384 B |

## Tick 9: All alerts together (WORST PERFORMANCE – most CPU cycles)

| Engine Temp | Battery | Speed | Tire Press. | Door | Seatbelt |
| --- | --- | --- | --- | --- | --- |
| 120.0 °C | 9.0 V | 130 km/h | 20 PSI | OPEN | UNLOCKED |

Active Alerts: 6  |  System Status: CRITICAL  |  Execution Time: 0.09 ms  |  Validation: PASS

[CRITICAL]  CRITICAL ENGINE OVERHEAT

[WARNING]  LOW BATTERY WARNING

[WARNING]  OVERSPEED WARNING

[WARNING]  LOW TIRE PRESSURE on FL

[WARNING]  DOOR OPEN WARNING

[WARNING]  SEATBELT WARNING

| Module | CPU Cycle | Code Size | Data Size |
| --- | --- | --- | --- |
| SensorManager | 1054 | 4 KB | 128 B |
| AlertManager | 2026 | 8 KB | 256 B |
| TOTAL | 3080 | 12 KB | 384 B |

## Tick 10: Boundary value verification

| Engine Temp | Battery | Speed | Tire Press. | Door | Seatbelt |
| --- | --- | --- | --- | --- | --- |
| 110.0 °C | 10.0 V | 120 km/h | 25 PSI | OPEN | UNLOCKED |

Active Alerts: 2  |  System Status: WARNING  |  Execution Time: 0.032 ms  |  Validation: PASS

[WARNING]  DOOR OPEN WARNING

[WARNING]  SEATBELT WARNING

| Module | CPU Cycle | Code Size | Data Size |
| --- | --- | --- | --- |
| SensorManager | 594 | 4 KB | 128 B |
| AlertManager | 814 | 8 KB | 256 B |
| TOTAL | 1408 | 12 KB | 384 B |

# 3.   Module-wise Aggregate Summary (Ticks 1–10)

| Module Name | Cycle Min | Cycle Max | Cycle Avg | Code Size | Data Size |
| --- | --- | --- | --- | --- | --- |
| SensorManager | 526 | 1054 | 695 | 4 KB | 128 B |
| AlertManager | 814 | 2026 | 1097 | 8 KB | 256 B |
| TOTAL | 1404 | 3080 | 1839 | 12 KB | 384 B |
| log_cycle_summary | NA | NA | NA | — | — |

| Metric | Value |
| --- | --- |
| Total Tests | 10 |
| Tests Passed | 10 (100.0%) |
| Total Execution Time | 0.418 ms |
| Average Time / Test | 0.042 ms |
| Fastest Test | Tick 4 – Overspeed (0.019 ms) |
| Slowest Test | Tick 9 – All Alerts Together (0.094 ms) |
| Best CPU Efficiency | Tick 7 – Low Tire Pressure (1,404 cycles) |
| Worst CPU Load | Tick 9 – All Alerts Together (3,080 cycles) |

# 4.   Justification of Performance Results

## Why Tick 9 (All Alerts Together) is Worst Case

Tick 9 records the highest CPU cycle count (3,080 total), making it the worst-case execution path. The following technical reasons explain this outcome:

- Maximum sensor evaluation overhead

All six sensor channels (engine temperature, battery, speed, tire pressure, door, seatbelt) simultaneously report fault conditions. Every check in the control pipeline executes without any early-exit opportunity.

- Alert cascade processing

Six concurrent alerts (1 CRITICAL + 5 WARNING) must each be individually detected, classified, registered, and written into the alert structure. Alert processing time scales linearly with alert count.

- CRITICAL state escalation logic

The presence of a CRITICAL alert (engine overheat at 120 °C) forces the system through the full escalation pathway: fault prioritisation → CRITICAL state evaluation → system lock checks. This adds significant overhead over warning-only paths.

- No early-exit optimisation possible

Unlike single-fault ticks where non-triggered paths are skipped, all conditional branches in SensorManager and AlertManager are taken, maximising the instruction count.

- AlertManager dominates at 2,026 cycles

With six active alerts, AlertManager alone consumes 2,026 of the 3,080 total cycles (65.8%), confirming that alert volume is the primary cost driver.

## Why Tick 7 (Low Tire Pressure) is Best Case

Tick 7 records the lowest CPU cycle count (1,404 total), making it the most efficient execution scenario. Key technical reasons include:

- Single, isolated fault condition

Only one sensor (tire pressure at 24.99 PSI, just below the 25 PSI threshold) triggers an alert. All other sensor values remain within normal range, allowing the majority of evaluation branches to exit early.

- Minimal AlertManager workload

A single WARNING alert requires only one detection, classification, and registration cycle. AlertManager completes in just 854 cycles — the lowest AlertManager count across all 10 ticks.

- No CRITICAL path traversal

The WARNING-level alert bypasses all CRITICAL state escalation logic, state lock evaluation, and priority arbitration code. These skipped paths represent a substantial saving in cycle count.

- SensorManager fast-path execution

With five of six sensors in normal range, SensorManager (550 cycles) completes minimal computation. Boundary checking for non-triggered sensors exits quickly without entering fault-registration routines.

- Stable, predictable execution pattern

A single warning condition creates a highly predictable branching pattern, enabling processor branch-prediction hardware to operate near-optimally and minimising pipeline stall penalties.

## Performance Comparison: Tick 7 vs. Tick 9

| Metric | Tick 7 – Best Case | Tick 9 – Worst Case |
| --- | --- | --- |
| Active Alerts | 1 (WARNING) | 6 (1 CRITICAL + 5 WARNING) |
| System Status | WARNING | CRITICAL |
| SensorManager Cycles | 550 | 1,054 |
| AlertManager Cycles | 854 | 2,026 |
| Total CPU Cycles | 1,404 | 3,080 |
| Execution Time | 0.029 ms | 0.090 ms |